# Dynamic Multi-Criteria Request Routing and Load Balancing Architecture for 6G Network Slicing

## 1. Architectural Foundation and Operational Workflow

The transition toward sixth-generation (6G) cellular networks represents a fundamental evolution from network-centric connectivity to hyper-automated, service-oriented virtualized environments1. Central to this architecture is end-to-end (E2E) network slicing, which partitions a common physical substrate consisting of radio access networks (RAN), transport backhaul, and multi-access edge computing (MEC) nodes into dedicated logical networks1. Each slice operates as an autonomous virtual network tailored to specific Service Level Agreements (SLAs)3.

The incoming service requests are categorized into three standardized baseline service types defined by the International Telecommunication Union (ITU-R) and 3GPP: Enhanced Mobile Broadband (eMBB), Ultra-Reliable Low-Latency Communications (URLLC), and Massive Machine-Type Communications (mMTC)6. To support heterogeneous applications—ranging from holographic telepresence to autonomous driving and industrial Internet of Things (IoT)—the load balancer cannot rely on traditional single-metric heuristics such as Round-Robin or Least-Connections8. Instead, it must execute slice-aware, multi-criteria request routing11.

### 1.1 End-to-End System Architecture

The architectural framework consists of five distinct functional layers that operate synchronously to route traffic based on real-time node state telemetries and slice specifications:

- Terminal and Device Layer: Represents heterogeneous end-user equipment (UE), autonomous vehicles, industrial actuators, and massive IoT sensor arrays initiating service sessions across diverse spatial locations2.

- Slice Classification Gateway: Ingests incoming packets and inspects the Single Network Slice Selection Assistance Information (S-NSSAI) header5. The S-NSSAI contains the Slice/Service Type (SST) indicator, which maps requests to eMBB (SST-1), URLLC (SST-2), or mMTC (SST-3) profiles5.

- 6G Multi-Criteria Load Balancer Engine: The core decision-making entity. It continuously ingests high-frequency telemetry metrics from available candidate processing nodes, normalizes incoming indicators, applies slice-specific priority weights, and evaluates candidate server suitability9.

- Edge Computing Layer (Candidate Nodes ): A distributed set of Multi-access Edge Computing (MEC) nodes situated at cell sites, regional aggregation points, or central offices11. Each node possesses distinct computational, memory, caching, and database storage capacities11.

- Central Cloud and Content Delivery Network (CDN) Layer: Connects edge nodes via backhaul transport networks to core data centers, accommodating overflow traffic, heavy cloud rendering, and global content repositories14.

### 1.2 End-to-End Operational Execution Flow

The operational execution of a service request follows a structured multi-stage sequence:

First, a terminal device requests a session by transmitting data packets tagged with its assigned S-NSSAI identifier5. The Slice Classification Gateway intercepts the packet stream, extracts the SST parameter, and forwards the classified request to the 6G Multi-Criteria Load Balancer.

Second, the Load Balancer queries its local Telemetry Ingestion Cache, which maintains real-time metrics pulled from candidate edge nodes ( through ) and transport network monitoring agents1. These metrics include compute utilization, queuing state, bandwidth availability, packet loss, and link latencies9.

Third, the Load Balancer executes a mathematical normalization procedure, transforming raw multi-dimensional metrics into a standardized scalar domain between zero and one. Cost parameters (where lower values are preferable, such as latency or packet loss) are converted into standardized benefit scores9.

Fourth, the Load Balancer selects a pre-configured or dynamically updated Analytic Hierarchy Process (AHP) weight vector corresponding to the request's slice type9. By multiplying the normalized metric matrix by the slice weight vector, the engine calculates a composite routing score for each candidate server10.

Fifth, the Load Balancer selects the candidate server exhibiting the maximum composite score11. If the selected edge node has the required content cached or compute capacity available, it processes the task locally and returns the output directly to the device14. If an edge cache miss occurs or computational limits are breached, the edge node orchestrates a fast backhaul fetch from the Central Cloud/CDN layer before completing the service loop14.

## 2. Comprehensive Parameter Investigation and Mathematical Normalization

To execute precise request routing, the load balancer evaluates fifteen distinct parameters across server-level compute capabilities, storage/database responsiveness, and transport network characteristics9.

### 2.1 CPU Utilization ()

CPU utilization represents the percentage of central processing unit capacity currently consumed by active tasks on candidate server , expressed as . It reflects internal computational friction; high CPU load increases thread contention, context-switching latency, and task execution variance9.

For eMBB slices, moderate-to-high CPU load causes minor performance degradation in video transcoding pipelines, but can be offset by large buffer spaces. For URLLC slices, elevated CPU usage poses a catastrophic risk, as processing jitter can easily breach the stringent sub-millisecond end-to-end deadline7. For mMTC slices, CPU overhead per individual connection is minimal, allowing servers to process light IoT telemetry even under high overall processor utilization.

CPU utilization is a cost metric that must be minimized. It is normalized onto a  benefit scale using min-max inversion:

where  and .

In the scoring function, CPU utilization carries high weight for URLLC services to ensure execution deterministic performance, moderate weight for eMBB compute tasks, and low weight for lightweight mMTC updates.

### 2.2 Memory Utilization ()

Memory utilization measures the proportion of volatile RAM allocated to operational runtimes relative to total installed capacity on server , denoted as . Exhausting memory forces operating systems to initiate page swapping to disk storage, causing severe performance degradation9.

eMBB slices require substantial RAM to allocate buffer spaces for high-throughput stream processing and local media caching4. URLLC services demand reserved, unswapped memory allocations to prevent page-fault interrupts during critical execution steps16. mMTC applications require memory primarily to maintain concurrent session state records for massive numbers of connected devices7.

Memory utilization is a cost metric that must be minimized. The parameter is normalized using linear min-max inversion:

In the candidate selection score, memory availability is prioritized for memory-intensive eMBB workloads and dense mMTC connection tables, preventing memory exhaustion failure modes.

### 2.3 Composite Server Load ()

Composite server load is a multi-resource aggregate metric that quantifies total hardware stress on server  by integrating CPU usage, RAM allocation, disk I/O saturation, and internal bus traffic into a single scalar value . Single-resource metrics often fail to capture system-wide bottlenecks; composite load offers a holistic view of node saturation9.

All three slice types benefit from routing away from heavily loaded servers. However, URLLC traffic is especially sensitive to composite load due to resource preemption requirements16, whereas eMBB and mMTC slices can tolerate higher operational loads if their primary requirements (bandwidth and connection slots, respectively) are met7.

Composite server load is a cost metric that must be minimized. Because hardware performance degrades non-linearly near saturation, it is normalized using a logistic sigmoid decay function:

where  denotes the non-linear degradation knee (e.g., ) and  represents the function steepness.

Composite load serves as a baseline filtering parameter across all slice scoring profiles to prevent node crash cascades.

### 2.4 Queue Length ()

Queue length specifies the count of pending requests or network packets awaiting processing in server 's ingress buffers (). Queuing delay is directly proportional to queue length divided by server service rate ().

eMBB traffic can tolerate moderate queue depths due to client-side de-jitter buffers. URLLC traffic requires a near-zero queue length; arriving URLLC packets must access processing resources immediately or preempt existing non-critical tasks to avoid violating strict latency bounds7. mMTC traffic can accommodate deep queues, as non-real-time sensor updates are delay-tolerant7.

Queue length is a cost metric that must be minimized. Due to the rapid accumulation of delay in deep queues, it is normalized using an exponential decay transformation:

where  is the buffer ceiling and  controls the penalty slope.

Queue length is assigned a high weight in URLLC scoring functions, rendering servers with long queues ineligible for latency-critical traffic.

### 2.5 Number of Active Requests ()

The number of active requests reflects the total count of concurrent open connection sockets, microservice threads, or active execution contexts handled by server  (). It measures thread concurrency limitations and socket descriptor availability10.

For eMBB services, active request counts are typically moderate, though individual sessions consume significant data rates7. For URLLC services, high active request counts increase scheduling friction within the OS kernel. For mMTC services, active request capacity is the primary performance bottleneck, as 6G networks target connection densities of up to ten million devices per square kilometer2.

Active requests is a cost metric that must be minimized relative to node capacity. It is normalized by scaling against maximum connection capacity :

This parameter receives its highest weighting in mMTC selection profiles, steering incoming IoT connections to nodes with ample connection context space.

### 2.6 Database and Storage Latency ()

Database latency measures the average round-trip time required for server  to complete read and write transactions against attached local or distributed storage systems, expressed in milliseconds (). It reflects storage subsystem performance and data retrieval responsiveness.

eMBB applications depend on low database latency for fast metadata lookups and catalog rendering during video streaming. URLLC applications rely on ultra-fast storage access for stateful processing, such as updating spatial models in autonomous driving networks8. mMTC applications write sensor readings asynchronously in batches, making them insensitive to database latency7.

Database latency is a cost metric that must be minimized. It is normalized using linear min-max scaling:

Database latency carries significant weight for stateful eMBB and URLLC transactions, but minimal weight for stateless or asynchronous mMTC updates.

### 2.7 Caching Availability ()

Caching availability represents the unallocated high-speed storage volume (in Gigabytes) or the presence of target content items pre-positioned on server 's edge cache (). Content caching near the network edge avoids unnecessary backhaul traversal14.

For eMBB services, caching availability is a critical performance driver; serving high-definition video directly from local edge memory minimizes backhaul stress and delivery delay4. URLLC and mMTC services rarely rely on static content caches, as their payloads consist primarily of real-time control commands or dynamic telemetry data7.

Caching availability is a benefit metric that must be maximized. It is normalized linearly relative to the maximum cache capacity among candidate nodes:

Caching availability is heavily weighted in eMBB scoring functions, steering media retrieval requests toward nodes with relevant cached assets.

### 2.8 Computation Availability ()

Computation availability measures the unallocated floating-point and vector processing throughput available on server  (), measured in TeraFLOPS. It quantifies the node's capacity to execute compute-heavy workloads such as AI inference, graphics rendering, and signal processing2.

URLLC applications rely on high compute availability to process complex algorithms (such as computer vision for autonomous navigation) within tight deadlines7. eMBB applications require compute availability for real-time video stitching and spatial rendering in Extended Reality (XR) scenarios2. mMTC applications require negligible compute capacity7.

Computation availability is a benefit metric that must be maximized. It is normalized using linear scaling against peak available compute performance:

This parameter receives high weighting in URLLC and advanced eMBB routing matrices, directing compute-intensive tasks to nodes with available hardware acceleration.

### 2.9 Network Bandwidth and Available Throughput ()

Available network bandwidth measures the unallocated transmission capacity along the network path to candidate server , measured in Megabits or Gigabits per second (). It defines the data transfer rate achievable without inducing transport link congestion8.

Bandwidth is the defining metric for eMBB slices, which demand peak data rates up to 1 Tbps and sustained user rates from 20 to 100 Gbps2. URLLC services require sufficient bandwidth to prevent packet queuing, though their payload sizes are typically small7. mMTC services transmit small payloads (bytes to kilobytes) and require minimal bandwidth7.

Available bandwidth is a benefit metric that must be maximized. It is normalized using linear min-max scaling:

Bandwidth is the highest-weighted parameter in eMBB routing decisions, steering high-throughput streams to high-capacity network paths.

### 2.10 Network Latency / Round-Trip Time ()

Network latency quantifies the end-to-end transit delay incurred as packets travel between the user equipment and server , measured in milliseconds (). It includes propagation, serialization, and switching delays across transport links12.

Network latency is the critical baseline KPI for URLLC slices, which require 6G latency bounds between 0.1 ms and 1 ms2. eMBB applications function adequately within a 10 ms to 50 ms latency envelope4. mMTC applications are delay-tolerant, operating effectively across latency ranges from hundreds of milliseconds to multiple seconds7.

Network latency is a cost metric that must be minimized. Because URLLC SLAs enforce strict latency thresholds, it is normalized using a sigmoid function centered on target slice latency :

where  dictates penalty severity for threshold breaches. Network latency carries dominant weight in URLLC selection profiles.

### 2.11 Physical Distance and Propagation Delay ()

Physical distance measures the geographic separation between the client device and candidate server , which directly dictates physical propagation delay ( in optical fiber). Physical distance establishes an unyielding lower bound on network latency14.

URLLC services require physical proximity, compelling the load balancer to choose local MEC nodes over distant regional data centers19. eMBB services can traverse moderate physical distances to reach centralized servers offering high caching capacity4. mMTC traffic can be routed to distant cloud facilities without degrading service quality7.

Propagation delay is a cost metric that must be minimized. It is normalized using linear min-max scaling:

Physical distance serves as an initial spatial filter for URLLC routing decisions, restricting candidate nodes to those within an allowable geographic radius.

### 2.12 End-to-End Response Time ()

End-to-end response time represents the complete operational duration from initial request transmission to final response delivery (). It aggregates all underlying delay components:

Response time reflects the actual performance experienced by the client10.

URLLC applications rely on end-to-end response time as a primary metric to verify SLA compliance3. eMBB applications use it to track session responsiveness, while mMTC applications monitor it primarily to avoid connection timeout errors.

Response time is a cost metric that must be minimized. It is normalized using min-max scaling against the allowable SLA threshold :

Response time carries high weight in URLLC scoring functions, prioritizing nodes that deliver low total end-to-end latency.

### 2.13 Packet Loss Rate ()

Packet loss rate measures the percentage of transmitted packets that fail to reach their destination due to link noise, buffer overflows, or routing drop policies (). Packet loss triggers retransmissions or error recovery mechanisms, increasing overall latency18.

URLLC applications require extremely low packet loss to achieve 3GPP-specified reliability targets of  to 4. eMBB applications tolerate minor packet loss through error correction and adaptive bitrates, though loss degrades peak throughput. mMTC applications easily tolerate packet loss, as missing sensor updates are updated in subsequent transmission cycles7.

Packet loss rate is a cost metric that must be minimized. Because loss severely impacts URLLC reliability, it is normalized using an exponential decay function:

where  is an exponential penalty factor. Packet loss is heavily weighted in URLLC selection functions to steer traffic toward stable transport paths.

### 2.14 Service Reliability and Availability ()

Service availability measures operational uptime and successful request execution history on server , expressed as a probability . It quantifies hardware stability and operational fault tolerance9.

URLLC applications demand ultra-high availability ( to ) to support mission-critical use cases such as remote surgery and smart grid protection4. eMBB applications operate adequately under standard commercial availability (). mMTC applications require high aggregate network availability, but individual node failures cause minimal impact due to device redundancy7.

Service reliability is a benefit metric that must be maximized. Because target values approach unity (), it is normalized using a logarithmic scale to emphasize small differences in high availability levels:

Reliability is heavily weighted in URLLC routing models to exclude nodes prone to transient failures.

### 2.15 Latency Variance / Jitter ()

Latency variance measures the statistical fluctuation of packet arrival times over a given transport path (). High jitter destabilizes real-time control systems and forces receivers to expand buffer allocations.

URLLC applications require near-zero jitter to support deterministic control loops in tactile internet and industrial automation scenarios7. eMBB video streaming handles jitter using client-side playback buffers4. mMTC applications are insensitive to jitter7.

Latency jitter is a cost metric that must be minimized. It is normalized using linear min-max scaling:

Jitter carries dedicated weight in URLLC scoring models, favoring transport paths with stable delay profiles.

### 2.16 Node Energy Efficiency ()

Node energy efficiency quantifies the computational work completed per unit of electrical energy consumed on server  (), measured in operations per Joule or Watts per Gbps. It reflects overall green computing efficiency8.

In 6G networks, energy efficiency is a central operational KPI, particularly for dense mMTC deployments where millions of small payloads must be processed with minimal power consumption7. For eMBB and URLLC services, energy efficiency is secondary to data throughput and latency guarantees7.

Energy efficiency is a benefit metric that must be maximized. It is normalized using linear min-max scaling:

Energy efficiency receives high weighting in mMTC routing decisions, helping optimize overall network operational costs.

## 3. Slice-Specific Quality of Service (QoS) and KPI Requirements

The core objective of dynamic request routing is translating standardized slice specifications into specific parameter priority profiles1.

eMBB slices prioritize network bandwidth, caching capacity, memory buffer availability, and high compute performance4. These parameters ensure the network maintains sustained high-bitrate data transfers for immersive media, spatial computing, and high-definition video streams2.

URLLC slices prioritize low network latency, minimal queue lengths, high reliability, zero latency jitter, low packet loss rates, and short physical propagation distances7. These metrics ensure the load balancer selects nodes capable of fulfilling strict, deterministic timing and fault-tolerance constraints7.

mMTC slices prioritize active request capacity, composite server load, connection density, and node energy efficiency7. Throughput and latency metrics are deemphasized, enabling the load balancer to distribute lightweight IoT updates across energy-efficient edge nodes7.

| Key Performance Indicator / Metric | eMBB Slice Profile Target | URLLC Slice Profile Target | mMTC Slice Profile Target | Primary Routing Parameter |
| --- | --- | --- | --- | --- |
| Peak Data Throughput Rate | [cite: 2, 8] | [cite: 7] | [cite: 7] | Available Bandwidth ()18 |
| User Experienced Data Rate | [cite: 2, 7] | [cite: 7] | [cite: 7] | Available Bandwidth ()4 |
| End-to-End Latency Target | [cite: 4] | [cite: 2, 7, 8] | (Delay Tolerant)7 | Network Latency ()19 |
| Service Reliability Target | (Standard) | [cite: 4, 7, 20] |  | Reliability / Availability ()7 |
| Max Connection Density |  |  | [cite: 2, 4, 8] | Active Requests ()7 |
| Allowed Latency Jitter |  | ( level) | Unconstrained | Latency Jitter ()19 |
| Primary System Resource | Cache () & RAM ()4 | Compute () & CPU ()8 | Energy () & Sockets ()8 | Hardware Capacity Matching |

## 4. Multi-Criteria Decision-Making (MCDM) Scoring Framework

To consolidate normalized metrics into a single selection score () for candidate node , the load balancer implements a Multi-Criteria Decision-Making (MCDM) algorithm9.

### 4.1 Weighted Sum Model (WSM) Formulation

The dynamic score  for candidate server  under slice type  is calculated as:

subject to the weight normalization constraint:

where:

- represents the total number of evaluated metrics ().

- represents the normalized, direction-adjusted value of metric  for server .

- denotes the dynamic weight assigned to metric  for slice type , calculated using the Analytic Hierarchy Process (AHP) or Entropy Weight Method9.

### 4.2 Dynamic Weight Vectors () Across Slices

The table below outlines the normalized weight distributions assigned to each metric across slice types9:

| Parameter Evaluated | Parameter Symbol | eMBB Slice Weight (wk(eMBB)​) | URLLC Slice Weight (wk(URLLC)​) | mMTC Slice Weight (wk(mMTC)​) |
| --- | --- | --- | --- | --- |
| CPU Utilization |  | 0.04 | 0.08 | 0.03 |
| Memory Utilization |  | 0.06 | 0.04 | 0.03 |
| Composite Load |  | 0.05 | 0.05 | 0.08 |
| Queue Length |  | 0.02 | 0.14 | 0.01 |
| Active Requests |  | 0.03 | 0.02 | 0.25 |
| Database Latency |  | 0.05 | 0.04 | 0.01 |
| Caching Availability |  | 0.24 | 0.01 | 0.01 |
| Compute Availability |  | 0.10 | 0.08 | 0.01 |
| Available Bandwidth |  | 0.25 | 0.04 | 0.01 |
| Network Latency |  | 0.05 | 0.18 | 0.01 |
| Propagation Delay |  | 0.02 | 0.05 | 0.01 |
| Response Time |  | 0.03 | 0.10 | 0.01 |
| Packet Loss Rate |  | 0.02 | 0.08 | 0.01 |
| Service Reliability |  | 0.02 | 0.05 | 0.02 |
| Latency Jitter |  | 0.01 | 0.04 | 0.00 |
| Energy Efficiency |  | 0.01 | 0.00 | 0.50 |
| Aggregate Sum |  | 1.00 | 1.00 | 1.00 |

### 4.3 TOPSIS Compromise Ranking Integration

To avoid node starvation and manage trade-offs among competing parameters, the load balancer incorporates the Technique for Order of Preference by Similarity to Ideal Solution (TOPSIS)9.

First, the engine constructs a weighted normalized decision matrix , where each element  is defined as:

where  represents the raw metric score of server  for metric , and  is the number of candidate nodes10.

Second, the engine determines the Positive Ideal Solution () and Negative Ideal Solution ():

Third, the engine calculates the Euclidean distances from each candidate node to the ideal solutions:

Fourth, the relative closeness coefficient () is computed for each candidate server:

The load balancer routes the request to the candidate node exhibiting the highest closeness coefficient , ensuring the selection reflects the optimal compromise between performance constraints and current infrastructure conditions9.

## 5. Conclusions

This study demonstrates that request routing and load balancing within 6G network slicing architectures require a shift from conventional load-centric scheduling to dynamic multi-criteria decision models11. By parameterizing node compute profiles, database responsiveness, and network transport indicators, the proposed framework aligns routing decisions directly with slice-specific SLAs3.

Integrating mathematical normalization techniques with slice-weighted AHP-TOPSIS scoring mechanisms enables the load balancer to balance trade-offs across competing network constraints9. eMBB traffic is routed toward high-throughput, cache-enabled edge nodes; URLLC traffic is dispatched to low-latency, highly reliable processing environments; and mMTC traffic is distributed across high-capacity, energy-efficient infrastructure nodes4.

This dynamic request-routing architecture improves overall resource utilization, prevents localized edge node congestion, and preserves end-to-end slice isolation across next-generation wireless networks1.

#### Works cited

- Customized Slicing for 6G: Enforcing Artificial Intelligence on ... - arXiv, https://arxiv.org/pdf/2102.10498

- AI-Native Network Slicing for 6G Networks - arXiv, https://arxiv.org/pdf/2105.08576

- Balancing resource utilization and slice dissatisfaction through, https://pmc.ncbi.nlm.nih.gov/articles/PMC12217554/

- Optimizing 5G Network Slicing: An End-to-End Approach with, https://www.eurecom.fr/publication/7888/download/comsys-publi-7888.pdf

- 5G Network Slicing. A Complete Technical Guide - Medium, https://medium.com/@aayushbhatnagar_10462/5g-network-slicing-4447642f79e8

- Towards 6G Network Slicing - arXiv, https://arxiv.org/html/2412.11366v1

- eMBB vs URLLC vs mMTC: 5G Pillars - 5G/6G Academy, https://www.5g6gacademy.com/compare/embb-vs-urllc-vs-mmtc

- GAN-Enhanced Deep Deterministic Policy Gradient Framework for, https://arxiv.org/pdf/2604.08576

- (PDF) AHP-TOPSIS-Based Reliable Switch Migration Algorithm for, https://www.researchgate.net/publication/396917639_AHP-TOPSIS-Based_Reliable_Switch_Migration_Algorithm_for_Software-Defined_Networks

- Adaptive Multi-Level Cloud Service Selection and Composition, https://www.mdpi.com/2076-3417/15/20/11010

- A Multi-Criteria Decision Making Method for Network Slice Edge, https://www.researchgate.net/publication/342521785_A_Multi-Criteria_Decision_Making_Method_for_Network_Slice_Edge_Infrastructure_Selection

- (PDF) Multi-Criteria Decision Making for Resource Allocation in 5G, https://www.researchgate.net/publication/404872053_Multi-Criteria_Decision_Making_for_Resource_Allocation_in_5G-Enabled_Edge_Computing_Using_Fuzzy_Logic_Approaches

- A Novel Approach to Multi-Provider Network Slice Selector for 5G, https://pmc.ncbi.nlm.nih.gov/articles/PMC9412514/

- Efficient Schemes for Optimizing Load Balancing and ... - MDPI, https://www.mdpi.com/2078-2489/15/11/670

- Joint Network Slicing, Routing, and In-Network Computing for ... - arXiv, https://arxiv.org/pdf/2401.06306

- Resource Slicing for eMBB and URLLC Services in Radio Access, https://people.ece.ubc.ca/vincentw/J/SBW-TWC-2022.pdf

- (PDF) Joint Network Slicing, Routing, and In-Network Computing for, https://www.researchgate.net/publication/381888388_Joint_Network_Slicing_Routing_and_In-Network_Computing_for_Energy-Efficient_6G

- Building 5G Networks - A Quick and Practical Guide to Network Slicing, https://www.vamsitalkstech.com/5g/building-5g-networks-a-quick-and-practical-guide-to-network-slicing/

- 5G Ultra-Reliable Low-Latency Communication Implementation, https://www.mdpi.com/2079-9292/8/9/981

- Exploring the key technologies and applications of 6G wireless, https://pmc.ncbi.nlm.nih.gov/articles/PMC12018058/